- Declaration of Performance

No. AUT-XXX-DOP-XXXX.XX.XX

| 1. | Unique identification code of the product-type: | Multi wall system chimney with ceramic liner |
| --- | --- | --- |
|  |  | Schiedel ADVANCE |
| 2. | Intended use: | Convey the products of combustion from heating appliances to the outside atmosphere |
| 3. | Product designation(s): | T200 N1 W 2 O00 |
| 4. | Manufacturer: | Schiedel GmbH, Friedrich-Schiedel-Str. 2-6, 4542 Nußbach Tel: 0043 50 6161-100, Fax: 0043 50 6161-111, info@schiedel.com |
| 5. | Authorized representative: | N/A |
| 6. | System(s) of AVCP: | System 2+ |
| 7. | Harmonized standard: | EN 13063-2:2005 + A1:2007 |
| . | Notified body: | 1085 |
| 8.. | European technical assesment | N/A |
| 9... | Declared performance | according the table ZA1 EN 13063-1:2005 + A1:2007 |

| Essential characteristics |  | Performance | Harm. technical specification |
| --- | --- | --- | --- |
| 5.2.5 | Resistance to fire external to external | NPD | EN 13063-2: 2005+A1:2007 |
| 5.2.1.3 | Resistance to fire internal to external/ Soot fire and thermal shock conditions | O00 |  |
| 5.3.1 | Gas tightness/leakage | N1 |  |
| 5.3.3 | Flow resistance of inner liners and fittings | r = 1,5 mm (ζ ≤ 1,6) |  |
| 5.2.4 | Dimensioning/Thermal resistance | DN 12 cm R68 DN 14 cm R62 DN 16 cm R53 DN 18 cm R66 DN 20 cm R52 DN 25 cm R85 |  |
| 5.2.1.3 | Thermal shock resistance | T200 |  |
| 5.1.2 | Compressive strength of clay/ceramic flue liners | ≥ 10 MN/m2 |  |
| 5.1.2 | Max. height of flue liner sections with openings | ≤ 25 m |  |
| 5.1.3.1.2 | Compressive strength of jointing material for flue liners | ≥ 10 N/mm2 |  |
| 5.1.5 | Compressive strength of outer wall elements | ≤ 35 m |  |

| Essential characteristics |  | Performance | Harm. technical specification |
| --- | --- | --- | --- |
| 5.1.7 | Compressive strength of jointing material for outer walls | ≥ 2,5 N/mm2 | EN 13063-2: 2005+A1:2007 |
| 5.3.2.1 5.3.2.2 | Durability / condensate resistance Corrosion resistance of flue liners and jointing material | Yes Yes |  |
| 5.5 | Freeze/thaw resistance | Yes |  |

10.	The performance of the product identified above is in conformity with the set of declared performance(s). This declaration of performance is issued, in accordance with Regulation (EU) No. 305/2011, under the sole responsibility of the manufacturer identified in point 4.

Signed for and on behalf of the manufacturer by:

Nußbach, 01.01.2021								Alessandro Cappellini

Geschäftsführer

……………………………..

(signature)

- Declaration of Performance

No. AUT-XXX-DOP-XXXX.XX.XX

| 1. | Unique identification code of the product-type: |  | Multi wall system chimney with ceramic liner |  |
| --- | --- | --- | --- | --- |
|  |  |  | Schiedel ABSOLUT |  |
| 2. | Intended use: |  | Chimneys |  |
| 3. | Product designation(s): | T200 N1 W 2 O00 |  |  |
| 4. | Manufacturer: |  | Schiedel GmbH, Friedrich-Schiedel-Str. 2-6, 4542 Nußbach Tel: 0043 50 6161-100, Fax: 0043 50 6161-111, info@schiedel.com |  |
| 5. | Authorized representative: |  | N/A |  |
| 6. | System(s) of AVCP: |  | System 2+ |  |
| 7. | Harmonized standard: |  | EN 13063-2:2005 + A1:2007 |  |
|  | Notified body: |  | 1085 | OFI CERT, OFI Technologie & Innovation GmbH, Franz Gril straße 5, A-1030 Wien |

8.	Declared performance according the table ZA1 EN 13063-1:2005 + A1:2007:

| Essential characteristics |  | Performance | Harm. technical specification |
| --- | --- | --- | --- |
| 5.2.5 | Resistance to fire external to external | NPD | EN 13063-1: 2006+A1:2007 |
| 5.2.1 | Resistance to fire internal to external | G 50 |  |
| 5.3.1 | Gas tightness/leakage | N1 |  |
| 5.3.3 | Flow resistance of inner liners and fittings | r = 1,5 mm (ζ ≤ 1,6) |  |
| 5.2.4 | Dimensioning/Thermal resistance | R40 [m2 K/W] |  |
| 5.1.2 | Max. height of flue liner sections with openings | ≤ 25 m / min. 50 kN |  |
| 5.1.3.1.2 | Compressive strength of jointing material for flue liners | ≥ 10 N/mm2 |  |
| 5.1.5 | Max. height of outer wall elements | ≤ 42 m |  |
| 5.1.6 | Compressive strength of jointing material for outer walls | ≥ 5 N/mm2 |  |
| 5.3.2 | Durability of gas tightness against chemicals/corrosion | Yes |  |
| 5.5 | Freeze/thaw resistance | Yes |  |

9.	The performance of the product identified above is in conformity with the set of declared performance(s). This declaration of performance is issued, in accordance with Regulation (EU) No. 305/2011, under the sole responsibility of the manufacturer identified in point 4.

Signed for and on behalf of the manufacturer by:

Nußbach, 01.01.2021								Alessandro Cappellini

Geschäftsführer

……………………………..

(signature)