- Declaration of Performance

No. DK-XXX-DOP-XXXX.XX.XX

| 1. | Unique identification code of the product type: | Single module concrete flue blocks |
| --- | --- | --- |
|  |  | Schiedel Isokern DM |
| 2. | Intended use: | Convey the products of combustion from heating appliances to the outside atmosphere |
| 3. | Product designation(s): | T450 N1 D 3 G00 |
| 4. | Manufacturer: | Schiedel Skorstene A/S, Industrivej 23, DK-7470 Karup Tel: 0045 70 10 20 11, |
| 5. | Authorized representative: | N/A |
| 6. | System(s) of AVCP: | System 2+ |
| 7. | Harmonized standard: | EN 1858-1:2008 + A1:2011 |
|  | Notified body: | 0036 |
| 8. | European technical assesment: | N/A |
| 9. | Declared performance: | according to the table ZA.1 EN 1858-1:2008 + A1:2011 |

| Essential characteristics |  | Performance | Harm. technical specification |
| --- | --- | --- | --- |
| 8.4 | Gas tightness | N1 | EN 1858-1:2008 +A1:2011 |
| 8.11.1 | Flow resistance of flue blocks | r = 3 mm |  |
| 8.11.2 | Flow resistance of fittings | NPD |  |
| 8.3 | Thermal resistance | 0,655 m2K/W |  |
| 8.1 | Resistance to fire / Heat stress resistance | T450 |  |
| 8.2 | Resistance to fire / Heat shock resistance | G00 |  |
| 8.13 | Fire resistance external to external | NPD |  |
| 4.2 | Reaction to fire | A1 |  |
| 8.6 | Compressive strength/max. building height | 30 m |  |
| 8.10 | Flexural strength under wind loading | 2,1 m |  |
| 8.8. | Condensate resistance | No, as designated D |  |
| 8.7 | Corrosion resistance | 3 |  |
| 8.5 | Abrasion resistance | Yes |  |
| 8.12 | Freeze/thaw resistance | NPD |  |
| 8.14 | Hazardous substances | None |  |

10.	The performance of the product identified above is in conformity with the set of declared performance(s). This declaration of performance is issued, in accordance with Regulation (EU) No. 305/2011, under the sole responsibility of the manufacturer identified in point 4.

Signed for and on behalf of the manufacturer by:

Karup, 0X.0X.20XX								Geir Vernan

General manager